- Instruction of DS1990 Copying Set

Description:

●Copy ROM(all types of iButton) to a SIMULATING REPLACEMENT.

●Test all types of Dallas iButton and display 64 bits-ROM.

Hardware:                 SET

Power

LCD Screen

Switch

＋

Buzzer

RESET

LCD

DOWN

C

D

E

F

UP

RS-232

8

9

A

B

DEL

4

5

6

7

ENTER

RJ45

0

1

2

3

BUTTON SOCKET

REPLACING PROBE/CLIPS

ATTENTION:

● “The system” presents your application system which is using the mother DS1990.

● “The mother DS1990” presents the iButton(DS1990) in your application system to be copied.

● REPLACING PROBE/CLIPS, which is used for connect the system to be copied with the SET when you use SIMULATING REPLACEMENT of DS1990. If you use clips (one red and one black) to connect the SET with the system, you ought to not use (or connect) the REPLACING PROBE/CLIPS.

1. Copy ROM (all types of iButton) to SIMULATING REPLACEMENT

Step1. Insert iButton (mother iButtton) to BUTTON SOCKET.

1. Demo          2. Crack

3. Copy           4. Type

Step2. Type 4 (4.Type)

DS1990

8D0000004927C601

Step3. Setting the number of SIMULATING REPLACEMENT

According to binary, SIMULATING REPLACEMENT can be set as 256 different numbers.

After setting, please record the number relative to which DS1990. For example, you want to set the number is 10100000, the setting is as follows:

1   2   3   4   5   6   7   8

Step4. Remove the mother iButtton, and then connect SIMULATING REPLACEMENT with the SET, that is, connecting the clips of SIMULATING REPLACEMENT to the clips of the SET, red to red and black to black.

DS1990

8D0000004927C601

Step5. Type 6

1. Demo          2. Crack

3. Copy           4. Type

Step6. Remove SIMULATING REPLACEMENT.

If you want to copy another one, you ought to set SIMULATING REPLACEMENT as different number for new DS1990, and then repeat.

2. Using SIMULATING REPLACEMENT

Step1. Set SIMULATING REPLACEMENT according to DS1990 being copied as above,

Step2. Make SIMULATING REPLACEMENT power on,

Step3. Insert SIMULATING REPLACEMENT into DS1990 socket of your system, and then press the button on SIMULATING REPLACEMENT.

Web site:  www.setchief.com    TEL/FAX:  0086-20-85662866    MOBILE:  13710680786    E-MAIL:   shichengdz@163.com      yang1006@21cn.com